# DRAFT REPORT: Implementation of Harmful Algal Bloom Algorithms for CE-QUAL-W2.

Prepared for: USACE-ERDC-EL-WQB

Prepared by: LimnoTech and Watercourse Engineering

Contents

Overview	1

Minimum Algae Concentration	2

Mechanical Removal (Treatment Regimes)	3

Assumptions	3

Modifications	3

Testing	9

Additional Notes	9

Hypoxic Conditions	11

Assumptions	12

Modifications	12

Testing	15

Additional Notes	15

Nitrogen Fixation	16

Assumptions	16

Modifications	16

Testing	18

Additional Notes	19

References	20

## Overview

Task 2 (“Implementation of Harmful Algal Bloom algorithms for CE-QUAL-W2”) of Delivery Order W912HZ23F0178 (“Water Quality Model Development to Manage Harmful Algal Blooms”) under USACE-ERDC Contract No. W912HZ-20-D-0004 called for LimnoTech and Watercourse Engineering to implement prioritized enhancements to improve W2’s utility for HABs applications.

Under Task 1 of this Delivery Order, LimnoTech and Watercourse Engineering worked with ERDC and other experts to identify and initially prioritize 21 opportunities for improvement in W2 capabilities with respect to HABs. Four code enhancement opportunities were selected for implementation in this task:

- Minimum Algae Concentration

- Mechanical Removal (Treatment Regimes)

- Hypoxic Conditions

- Nitrogen Fixation

This document provides additional details about the code enhancements, including the purpose of the enhancement, descriptions of modifications to input requirements and calculation code, initial verification testing, and additional notes.

As the enhancements are theoretical representations based on conceptual models of processes and on behavior imputed from field observations, model studies would be a prudent next step to comprehensively test the power and validation of the new enhancements. The verification testing of the new code presented below is intended to demonstrate proper behavior of the new processes’ mathematical representation such conservation of mass, etc.

## Minimum Algae Concentration

Goal:  Enforce a minimum non-zero algae concentration so that recovery occurs without adding biomass through boundary conditions.

This enhancement may raise some issues with respect to mass balance, but is a necessary option for addressing high algae mortality impacts under hypoxic conditions. The enhancement will support reasonable model predictions for post low-DO recovery of algae levels, and is therefore addressed below with the Hypoxic Conditions enhancement.

## Mechanical Removal (Treatment Regimes)

Goal: Add capability to remove biomass in one or more segments.

### Assumptions

- Removal occurs instantaneously at a specified time step in specified segments.

- Removal operations can be repeated at the same location.

- Algae are removed from the surface to a specified depth.

- If the specified depth is in the middle of a layer, the depth is set to the depth of the bottom of the layer.

- partial layer height).

- Removal effectiveness is constant over the entire removal depth.

### Modifications

This section describes the implementation of the Mechanical Removal capability.

Notation. The W2 model stores all water quality constituent concentrations in one array and assigns pointers with meaningful names to appropriate sections. Algae state variable concentrations are referenced as follows:

! State variables (Pointers to data assigned here)

ALG  => C2(:,:,NAS:NAE)   !NAS is location of 1st algae type simulated, NAE = NAS+NAL is last type

New Variables. New variables were added to CE-QUAL-W2 (v4.5) to:

- determine if an input file describing algal harvesting exists and read it if it does.

- specify time-varying fractional reductions on a segment-specific basis to reflect the impact of mechanical harvesting activity on algal group concentrations.

The variables were added to the code as shown in Table 1. The variables are declared in modules GLOBAL, TVDC, LOGICC, and MAIN where variables used for similar time-varying inputs (e.g., segment water withdrawals) are declared in the original code. New allocatable arrays are allocated, and new input variables are read. The algal harvesting option was implemented in the WQCONSTITUENTS subroutine.

Table 1. New Variables for Mechanical Removal.

| Name | units | Description |
| --- | --- | --- |
| Harvest activation: HARVEST_OPTION_EXIST | --- | Read the “harvest_option_input.csv” input file Logic switch indicating whether the algal harvesting input file “harvest_option_input.csv” exists, or not. |
| HAFFN | --- | Name of file that contains time-varying harvesting inputs. |
| NHF | --- | Number of segments with time-varying harvesting inputs. |
| IHA(1 … NHF) | --- | Segment numbers with time-varying harvesting inputs. |
| HAIC(1 … NHF) | --- | Logic switch (1=true, 0=false) for linear interpolation. |
| Harvest time-series: HAFF | --- | Read the HAFFN input file Logic switch (a “$” in column 1 of the first line of HAFFN indicates the file is cvs-format). |
| NXFHA1, NXFHA2 | day | Simulation day (relative to JDAY) to apply harvesting |
| FHANX(1 to NHF) | fraction | Segment-specific fraction of algae removed by harvesting. |

Code modifications. The code changes for the algal harvesting capability are implemented without changing the W2 model input control file (i.e., w2_con.npt or w2_con.csv). A separate comma-separated value input file (“harvest_option_input.csv”) controlling this new capability is read by the model if the file exists in the same folder as the W2 model input control file. The inputs allow the user to specify where (i.e., segment locations) and how (i.e., interpolate or not) harvesting is applied and the name of the file (HAFFN) describing the time-varying fractional harvest reduction forcing conditions. The time-varying harvest reductions are implemented and applied in a manner that follows other existing W2 model time-varying input forcings (e.g., water withdrawals from segments).

The general format for the nine-line “harvest_option_input.csv” input file (including header lines that can be used to describe the input variables and/or other user-specified information) is as follows for an example with 11 segments being harvested:

Algal harvesting activation inputs

Name of file with time-varying algal harvesting inputs

algal_harvesting_time-series.csv

NHF: the number of segments with time-varying harvesting

11

IHA(1) to IHA(NHF): List of segments with harvesting

11, 12, 13, 44, 45, 56, 57, 58, 59, 60, 61

HAIC(1) to HAIC(NHF): Linear interpolation (0=NO, 1=YES)

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

The name of the time-varying harvest reductions FAFFN input file is specified in the “harvest_option_input.csv” input file. This file may be either fixed- or csv-format, consistent with other W2 model time-varying data input files.

The .csv format version of the FAFFN input file is as follows for varying algal reductions starting at various simulation times and lasting for 1 day (and with no linear interpolation), including however many input lines are needed to describe the simulation time and corresponding fractional reductions in algae imposed for segments being harvested over the course of the model simulation period:

$ NFXHA, FHANX(1 to NHF): JDAY and removal fractions for NHF=11.

1.0,   0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

159.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

160.0, 0.7, 0.7, 0.7, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

161.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

199.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

200.0, 0.0, 0.0, 0.0, 0.9, 0.9, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

201.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

229.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

230.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.8, 0.8, 0.8, 0.8, 0.8, 0.8

231.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

1000., 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0

The fixed-format version of the FAFFN input file is as follows for varying algal reductions starting at various simulation times and lasting for 1 day (and with no linear interpolation), including however many input lines are needed to describe the simulation time and corresponding fractional reductions in algae imposed for segments being harvested, over the course of the model simulation period:

NFXHA, FHANX(1 to NHF): JDAY and removal fractions for NHF=11.

1.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

159.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

160.0     0.8     0.8     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

161.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

199.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

200.0     0.0     0.0     0.8     0.8     0.8     0.0     0.0     0.0     0.0

0.0     0.0

201.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

229.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

230.0     0.0     0.0     0.0     0.0     0.0     0.8     0.8     0.8     0.8

0.8     0.8

231.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

1000.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0     0.0

0.0     0.0

The code changes implemented for the algal harvesting option are presented below.

(Note: the code shown excludes end-of-line comments and consolidates extra space, as needed, to fit within the margins of this document. Line numbers for each of the revised code files are shown for the modified/added code shown below.)

Declare new variables in code file  w2modules.f90  –

Module GLOBAL:

(112)	INTEGER                                   :: NHF

Module TVDC:

(204)	REAL(R8),     ALLOCATABLE, DIMENSION(:)   :: FHA,    FHASAV

(210)	CHARACTER(72)                             :: HAFFN

Module LOGICC:

(426)	LOGICAL                                   :: HARVESTING

(439)	LOGICAL,      ALLOCATABLE, DIMENSION(:)   :: INTERP_HARVESTING

Module MAIN:

(576)	INTEGER       :: JHA

(614)	LOGICAL       :: HARVEST_OPTION_EXIST

(635)	REAL(R8),   ALLOCATABLE, DIMENSION(:)   :: FHAO

(680)	INTEGER,    ALLOCATABLE, DIMENSION(:)   :: IHA,   JBHA,  HAIC

Allocate and apply new variables in code file input.f90 –

Allocate memory for arrays

(197)	ALLOCATE (HAIC(IMX))

(312)	ALLOCATE (FHAO(IMX))

(426)	ALLOCATE (JBHA(IMX), FHA(IMX), FHASAV(IMX), INTERP_HARVESTING(IMX))

Determine and/or read new variables in the “harvest_option_input.csv” input file

(2332)	!> sch 28Jan2025. Start algal harvesting option input reading ...

(2333)	 HARVEST_OPTION_EXIST = .FALSE.

(2334)	 NHF = 0

(2335)	 INQUIRE(FILE='harvest_option_input.csv',EXIST=HARVEST_OPTION_EXIST)

(2336)	 IF(HARVEST_OPTION_EXIST) THEN

(2337)	   OPEN(NUNIT,file='harvest_option_input.csv',status='old')

(2338)	   READ (NUNIT, *)

(2339)	   READ (NUNIT, *)

(2340)	   READ (NUNIT, *) HAFFN

(2341)	   READ (NUNIT, *)

(2342)	   READ (NUNIT, *) NHF

(2343)	   READ (NUNIT, *)

(2344)	   READ (NUNIT, *) (IHA(I), I=1,NHF)

(2345)	   READ (NUNIT, *)

(2346)	   READ (NUNIT, *) (HAIC(I), I=1,NHF)

(2347)	   CLOSE(NUNIT)

(2348)	 ENDIF

(2349)	!> sch 28Jan2025. End algal harvesting option input reading.

Determine model grid locations for harvesting in code file init-geom.f90 –

(252)	!> sch 29Jan2025. Start algal harvesting option.

(253)	  IF (HARVESTING) THEN

(254)	    DO JHA=1,NHF

(255)	      IF (IHA(JHA) >= US(JB).AND.IHA(JHA) <= DS(JB)) JBHA(JHA) = JB

(256)	    END DO

(257)	  END IF

(258)	!> sch 29Jan2025. End algal harvesting option.

Initialize certain harvesting-related variables in code file init.f90 –

(40)	FHA    = 0.0D0;  FHASAV = 0.0D0

(101)	HARVESTING            =  NHF > 0

(109)	INTERP_HARVESTING  = HAIC   == 1

Read and process time-varying harvesting inputs in code file time-varying-data.f90 –

Define / allocate and initialize harvesting option variables

(20)	REAL                                   :: NXFHA1, NXFHA2

(28)	REAL(R8),ALLOCATABLE, DIMENSION(:)     :: FHAO, FHANX

(44)	INTEGER                                :: HAF, JHA

(49)	LOGICAL                                :: HAFF

(96)	ALLOCATE (FHAO(NHF), FHANX(NHF))

(139)	HAFF = .FALSE.

Initial reading of time-varying harvesting inputs

(470)	!> sch 29Jan2025. Start algal harvesting option. ...

(471)	  IF (NHF > 0) THEN

(472)	    HAF = NPT; NPT = NPT+1

(473)	    OPEN (HAF,FILE=HAFFN,STATUS='OLD')

(474)	    READ (HAF,'(A1)') INFORMAT

(475)	    IF(INFORMAT=='$') HAFF=.TRUE.

(476)	    IF(HAFF) THEN

(477)	    READ (HAF,'(/)')

(478)	    READ (HAF,*) NXFHA2,(FHANX(JHA),JHA=1,NHF)

(479)	    DO JHA = 1,NHF

(480)	      FHA(JHA)  = FHANX(JHA)

(481)	      FHAO(JHA) = FHANX(JHA)

(482)	    END DO

(483)	    READ (HAF,*) NXFHA1,(FHANX(JHA),JHA=1,NHF)

(484)	    ELSE

(485)	    READ (HAF,'(//10F8.0:/(8X,9F8.0))') NXFHA2,(FHANX(JHA),JHA=1,NHF)

(486)	    DO JHA = 1,NHF

(487)	      FHA(JHA)  = FHANX(JHA)

(488)	      FHAO(JHA) = FHANX(JHA)

(489)	    END DO

(490)	    READ (HAF,'(10F8.0:/(8X,9F8.0))') NXFHA1,(FHANX(JHA),JHA=1,NHF)

(491)	    ENDIF

(492)	    FHASAV(1:NHF) = FHA(1:NHF)

(493)	  END IF

(494)	!> sch 29Jan2025. End reading algal harvesting option inputs.

Continued reading of time-varying harvesting inputs

(1710)	!> sch 29Jan2025. Start algal harvesting option. ...

(1711)	  IF (NHF > 0) THEN

(1712)	    DO WHILE (JDAY >= NXFHA1)

(1713)	      NXFHA2 = NXFHA1

(1714)	      DO JHA=1,NHF

(1715)	        FHA(JHA)  = FHANX(JHA)

(1716)	        FHAO(JHA) = FHANX(JHA)

(1717)	      END DO

(1718)	      IF(HAFF)THEN

(1719)	      READ (HAF,*) NXFHA1,(FHANX(JHA),JHA=1,NHF)

(1720)	      ELSE

(1721)	      READ (HAF,'(10F8.0:/(8X,9F8.0))')NXFHA1,(FHANX(JHA), JHA=1,NHF)

(1722)	      ENDIF

(1723)	      FHASAV(1:NHF) = FHA(1:NHF)

(1724)	    END DO

(1725)	    NXTVD = MIN(NXTVD,NXFHA1)

(1726)	  END IF

(1727)	!> sch 29Jan2025. End algal harvesting option.

Apply linear interpolation if that user-input option is specified

(2871)	!> sch 29Jan2025. Start algal harvesting option. Interpolate ...

(2872)	  IF (NHF > 0) THEN

(2873)	    QRATIO = (NXFHA1-JDAY)/(NXFHA1-NXFHA2)

(2874)	    DO JHA=1,NHF

(2875)	      IF (INTERP_HARVESTING(JHA)) THEN

(2876)	        FHA(JHA) = (1.0-QRATIO)*FHANX(JHA)+QRATIO*FHAO(JHA)

(2877)	        FHASAV(JHA) = FHA(JHA)

(2878)	      END IF

(2879)	    END DO

(2880)	  END IF

(2881)	!> sch 29Jan2025. End algal harvesting option.

Deallocate memory for local variables

(3059)	DEALLOCATE (FHAO, FHANX)

Implement algal harvesting impact on algae in “wqconstiTuents.f90” –

Apply harvest reduction fractions to the harvesting segments and the simulated algal groups.

(226)	!> sch 29Jan2025. Start algal harvesting option

(227)	IF (HARVESTING) THEN

(228)	 DO JHA=1,NHF

(229)	  IF (FHA(JHA) /= 0.0) THEN

(230)	   IF (JB == JBHA(JHA)) THEN

(231)	    I = MAX(CUS(JBHA(JHA)),IHA(JHA))

(232)	    DO K=KTW(JHA),KBW(JHA)

(233)	     IF (JC >= NAS .AND. JC <= NAE) CSSB(K,I,JC) = CSSB(K,I,JC) * FHA(JHA)

(234)	    END DO

(235)	   END IF

(236)	  END IF

(237)	 END DO

(238)	END IF

(239)	!> sch 29Jan2025. End algal harvesting option

Deallocate memory associated with new variables in code file endsimulation.f90 –

(273)	DEALLOCATE (FHASAV)

(280)	DEALLOCATE (FHAO)

(321)	DEALLOCATE (JBHA,   IHA,    FHA)

(343)	DEALLOCATE (INTERP_HARVESTING)

### Testing

Demonstration of expected change in result from enabling feature.

### Additional Notes

- This initial implementation only considers removal of living algae (algal state variables), not POM (aka not living) (separate non-algal state variable)

- Notes from earlier review of CE-QUAL-W2 handling of added sub-models or forcing conditions as these relate to incorporating algae harvesting as an option (Scott Hinz, 25Oct2024):

  - ON/OFF switch variables in the W2 model input control file’s (W2_con.npt) Miscellaneous (MISCELL) input card/block represent how the options for simulating sediment diagenesis and hypolimnetic oxygen injection are activated, including the reading of separate external input files describing how the model will incorporate these processes during a simulation. Algae harvesting can be initiated in a W2 simulation using this input card by adding an ON/OFF switch variable for this forcing condition. Might be a good approach

  - In general, time series forcings will go into (or out of) effect within a time step of the JDAY (model simulation timestamp) specified by user inputs, since nearly all W2 model applications will employ variable time-stepping for computational efficiency reasons. The worst-case precision is dependent on the user-specified maximum timestep, but the precision will vary throughout a model simulation simply because calculation time step can increase or decrease significantly due to tradeoffs between computational efficiency and model-estimated numerical stability.

  - The file structure for more recent options is commonly implemented for csv input file formats, but a fixed format input option can also be implemented if desired as an option (but will obviously not conform to the W2 model’s Excel-based input templates). Some guidelines for a csv input file structure are provided in Part 3 of the W2 user manual. Start simple!

  - The hypolimnetic oxygen injection source code (file “aerate.f90”) appears to be a relevant template for coding algae harvesting. The oxygen injection is read in as a time series forcing condition for adding mass to specified water column layers of user-specified segments and it impacts (by addition) the predicted mass of the DO state variable. Algae mass removal represents the opposite impact (by subtraction) but on an algae state variable (or for multiple algae state variables). The oxygen injection source code includes the required logic for identifying the model grid waterbody and branch for a segment specified to receive oxygen mass injection, providing an additional reason for why this code can serve as a general template for implementing algae harvesting as an optional capability … even if the information input to calculate the mass removal is likely to be somewhat different (e.g., percent mass removal vs. explicit mass addition).

  - Based on the hypolimnetic oxygen injection code implementation, the following files would be edited and/or created to incorporate an algae harvesting option:

    - algae-harvest.f90 – new source code (using whatever filename is desired) for harvesting calculations, making use of “areate.f90” as a general template.

    - input.f90 – reads run control input file “w2_con.npt” and where an input variable switch for algae harvesting would need to be implemented.

    - w2modules.f90 – non-local variable definitions and memory allocation

    - wqconstituents.f90 –  water quality calculations are done here and this is where state variable mass source and sink terms are accounted for during a simulation.

    - endsimulation.f90 – end of simulation memory deallocation

    - w2_4_win.f90 – this is a “probably not” (i.e., no changes) for algae harvesting. For oxygen mass injection code in here is needed because altering vertical mixing is an input option.

## Hypoxic Conditions

Goal: Provide a low dissolved oxygen (DO)-high algal mortality option for algae kinetics.

Discussion: Low DO can impede several biological processes. For example, nitrification is inhibited under diminished oxygen concentrations as is organic matter decay. In these cases, low DO inhibits aerobic bacterial activity. For nitrification, these include conversion of ammonia to nitrite (e.g., Nitrosomonas) and nitrite to nitrate (Nitrobacter). For organic material there are a wide range of aerobic genera that consume oxygen in the decay of organic matter, and when oxygen levels drop to low levels there are anaerobic genera that contribute, typically at a lower rate.

Algal respiration is another oxygen demand on aquatic systems, often ignored in models because photosynthesis of these organisms is typically much larger. However, large concentrations of algae exhibiting extreme diel dissolved oxygen ranges can result in “crashes” in hypereutrophic systems (commonly inhabited by HABS). Imposing immediate and significant oxygen demands on the water body, particularly in shallower systems where settled material from the bloom continue to exert oxygen demands on the water column. Persistent anoxia can thus impede the recovery of algal populations because nighttime algal respiration demands exceed available oxygen.  Including such logic in the CE-QUAL-W2 model extends the range of applications for the model, particularly in simulating HABS. A high algal mortality rate in response to ongoing low DO conditions can be used to represent these effects in the model.

The logic behind this kinetic option assumes that physiological changes include increased respiration, reduced nutrient (e.g., N, P, C, Si) uptake, and other physiological stresses. The impacts of low DO can then be manifest in the algal population as an increase in the mortality rate.

Onset of low DO impacts can be developed as a

- simple limit (e.g., DO below DOcrit increases mortality by a factor or to a set value),

- linear decrease below DOcrit to zero over time ,

- Monod kinetics where the increase in mortality follow a Michalis-Menton representation, or

- other

The first option is proposed but also incorporates a low DO conditions criteria for triggering a high algal mortality rate

### Assumptions

- All algae in the segment layer will be equally subject to the low DO criteria and associated increased mortality

- DO can recover and subsequently degrade to adverse conditions multiple times

- Increased mortality may lead to algae concentration dropping to a user specified minimum

### Modifications

A previous W2 v3.1 implementation of low DO-high mortality is documented in a draft technical memorandum (Watercourse Engineering 2005) and was used as a guide for W2 v4.5 code implementation.

Notation. The W2 model stores all water quality constituent concentrations in one array and assigns pointers with meaningful names to appropriate sections. The following describes how algae and DO state variable concentrations are referenced.

! State variables (Pointers to data assigned here)

ALG  => C2(:,:,NAS:NAE)   !NAS is location of 1st algae type simulated, NAE = NAS+NAL is last type

O2   => C2(:,:,NDO)

Table 2. New Variables for Low DO Impacts.

| Name | units | Description |
| --- | --- | --- |
| ALG_O2LIM | mg/L | [O2] below which, if sustained for CRIT_T, high mortality (AM_LOW_DO) sets in. |
| ALG_MIN | mg/L | Minimum [ALG] standing crop (thus can retain a small standing crop) |
| DO4 | --- | A kind of switch-multiplier set according to whether [O2] is below ALG_O2LIM. In style of existing code. |
| DELT_LOW_DO | sec | Duration of time during which [O2] below ALG_O2LIM. (Calculated within code.) |
| CRIT_T | days | Duration of low DO (ALG_O2LIM) after which high mortality sets in. |
| AM_LOW_DO | 1/day | High mortality rate associated with low DO conditions |

New Variables. Six variables were added to CE-QUAL-W2 (v4.5) to ensure a minimum standing crop of algae and to set algae mortality high during extended periods of low DO. Each is declared in the KINETICS and MAIN modules where similar variables are declared in the unmodified code.  New allocatable arrays are allocated, and new variables are read.  High algae mortality logic is implemented in the KINETICS subroutine.

The changes for the low DO-high algal mortality capability were implemented to not impact the W2 model input control file (i.e., w2_con.npt or w2_con.csv). A separate comma-separated value input file (“low_do_mort_input.csv”) controlling this logic is read by the model if the file exists in the same folder as the W2 model input control file. The inputs allow the user to specify values for ALG_O2LIM, ALG_MIN, CRIT_T, and AM_LOW_DO.

The content of the input file (including header lines that are used to describe the input variables and/or other user-specified information) is as follows:

Low DO – high algal mortality inputs for 1 to NAL algal groups [header]

ALG_O2LIM (mg/L), ALG_MIN (ug Chl-a/L) [descriptive info]

1.0, 0.1

CRIT_T (days), 1 to NAL [descriptive info]

1.0, 2.0, ... 3.0

AM_LOW_DO (1/day), 1 to NAL [descriptive info]

1.0, 1.0, ... 1.0

New Code. The code changes implemented for the low DO-high mortality option are shown below. Code line numbers for each of the revised code files precede the code changes listed below, and end-of-line comments are omitted.

If DELT_LO_DO < CRIT_T and ALG > ALG_MIN, the mortality rate (AMR) is set to AM_LOW_DO; otherwise the base input rate for mortality is maintained. Since AMR is redetermined based on the input mortality rate and calculated water temperature upon the next timestep for each segment and layer, there is no need to reset to the base level of mortality once DO concentrations rise above ALG_O2MIN.

Declare new variables in code file w2modules.f90 –

Module Kinetic:

(220)	REAL                                :: O2LIM, ALG_O2LIM, ALG_MIN

(312)	REAL,  ALLOCATABLE, DIMENSION(:,:)  :: DO4,    DELT_LOW_DO

(313)	REAL,  ALLOCATABLE, DIMENSION(:)    :: CRIT_T, AM_LOW_DO

Module Main:

(612)	LOGICAL       :: LOW_DO_MORTALITY_EXIST

Allocate and apply new variables in code file input.f90 –

Allocate memory for arrays

(327)	ALLOCATE (CRIT_T(NAL),  AM_LOW_DO(NAL))

(344)	ALLOCATE (DO4(KMX,IMX),  DELT_LOW_DO(KMX,IMX))

Read in new variables in the “low_do_mort_input.csv” input file

(2299)	!> sch 25Jan2025. Start Low DO - high mortality option input reading.

(2300)	  LOW_DO_MORTALITY_EXIST = .FALSE.

(2301)	  DELT_LOW_DO = 0.0

(2302)	  INQUIRE(FILE='low_do_mort_input.csv',EXIST=LOW_DO_MORTALITY_EXIST)

(2303)	  IF(LOW_DO_MORTALITY_EXIST) THEN

(2304)	    OPEN(NUNIT,file='low_do_mort_input.csv',status='old')

(2305)	    READ (NUNIT, *)

(2306)	    READ (NUNIT, *)

(2307)	    READ (NUNIT, *) ALG_O2LIM, ALG_MIN

(2308)	    READ (NUNIT, *)

(2309)	    READ (NUNIT, *) (CRIT_T(JA), JA=1,NAL)

(2310)	    READ (NUNIT, *)

(2311)	    READ (NUNIT, *) (AM_LOW_DO(JA), JA=1,NAL)

(2312)	    CLOSE(NUNIT)

(2313)	    AM_LOW_DO = AM_LOW_DO / DAY

(2314)	    CRIT_T = CRIT_T * DAY

(2315)	  ENDIF

(2317)	!> sch 25Jan2025. End Low DO - high mortality option input reading.

Update variables in “update.f90” to ensure a nominal standing crop of algae –

Ensure a nominal standing crop of algae is maintained

(94)	  DO J=1,NAL

(95)	     IF(ALG(K,I,J)<ALG_MIN) ALG(K,I,J)=ALG_MIN

(96)	  END DO

Implement low DO – high mortality in “water-quality.f90” subroutine KINETICS –

Determine mortality switch/multiplier and record duration of low DO

(278)	  DO4(K,I)    = (1.0+SIGN(1.0,ALG_O2LIM  -O2(K,I)))*0.5

(279)	  DELT_LOW_DO(K,I)=(DELT_LOW_DO(K,I)+DLT)*DO4(K,I)

Reset mortality in all layers

(676)	 IF(DELT_LOW_DO(K,I)>CRIT_T(JA)) AMR(K,I,JA) = AM_LOW_DO(JA)

Deallocate memory associated with new variables in code file endsimulation.f90 –

(304)	DEALLOCATE (DO4,    DELT_LOW_DO,    CRIT_T, AM_LOW_DO)

### Testing

Demonstration of expected change in result from enabling feature.

### Additional Notes

- Code for inputs - the .csv and .npt input structure is different (e.g., the column-row structure is reversed) for algal kinetic parameters, so feedback on a preferred approach for low DO-high mortality inputs is important prior to implementing this capability in the W2 code.

- The previous W2 v3.1 implementation of low DO-high mortality is documented in a draft technical memorandum (Watercourse Engineering 2005) and can be used as a guide for W2 v4.5 code implementation. The following files (no new code files are needed) should  be edited and/or created to incorporate this capability:

  - input.f90 – reads run control input files “w2_ con.csv” or “w2_con.npt” and where the input variables (assuming a kinetics-modification approach is most appropriate in this case) needed to control this capability are read.

  - w2modules.f90 – non-local variable definitions and memory allocation

  - water-quality.f90 –  add code for low DO-mortality calculations (see 2005 Watercourse Engineering memo).

  - wqconstituents.f90 – add code to set a minimum algae standing crop (see 2005 Watercourse Engineering memo).

  - endsimulation.f90 – end of simulation memory deallocation.

  - Various time variable and snapshot option output code files (output.f90, outputa2wtools.f90 and outputinitw2tools.f90)  – will be modified for tracking when, where and duration of increased mortality (for each specific algae group to which it may be applied).

## Nitrogen Fixation

Goal: Incorporate a base level of nitrogen (N)-fixation regardless of the total inorganic nitrogen (TIN) concentration that is available to support algal growth.

N-fixation occurs when certain cyanobacteria (there are other algae that can utilize N2) utilize N2(aq) in water. N2(aq) has a low solubility in water and at STP equilibrium concentration is approximately 20 mg/L. The assumption is that N2(aq) is not limiting.

### Assumptions

The assumptions behind incorporating an optional base level of N-fixation in the W2 model are that:

- N-fixation occurs when diazotrophic (N-fixing) cyanobacteria species are present

- Certain forms may have a base level of fixation regardless of TIN concentration. One way to envision this is chronic N-limitation which can be a function of cyanobacteria species, morphology (e.g., colony forming), environment, and other factors.

- Certain cyanobacteria species will “switch” to nitrogen fixation at a specific TIN level. However, local TIN levels will vary on a micro-scale and thus the entire water body (or model segment layer) may not “look” binary (on or off). Further, different cyanobacteria species may commence fixation at different TIN levels.

### Modifications

Changes were made to CE-QUAL-W2 (W2) v4.5 to implement simulation of an alternative nitrogen fixation (N-fixation) option for algal growth that allows for N-fixation to occur when available total inorganic nitrogen (TIN) concentrations (i.e., the sum of dissolved ammonia and nitrite+nitrate nitrogen) fall below a user-specified critical value, instead of requiring TIN levels to effectively drop to zero before N-fixation can occur.

Notation. The W2 model stores all water quality constituent concentrations in one array and assigns pointers with meaningful names to appropriate sections. The following describes how algae state variable concentrations are referenced.

! State variables (Pointers to data assigned here)

ALG  => C2(:,:,NAS:NAE)   !NAS is location of 1st algae type simulated, NAE = NAS+NAL is last type

New Variables. Baseline N-fixation capability is assumed to be a function of water column algal species. Since an N-uptake half-saturation coefficient is already input to the model, the only proposed additional algal group-specific input parameter is a switch to identify that an algal group has a base level of N-fixation because it is a type of diazotrophic cyanobacteria. The variables added to CE-QUAL-W2 (v4.5):

- determine if an input file describing N-fixation exists and read it if it does.

- set a critical low concentration for TIN as a trigger for initiating N-fixation.

These variables are declared in the KINETICS and MAIN modules where similar variables are declared in the unmodified code.  New allocatable arrays are allocated, and new variables are read. The N-fixation option is implemented in the KINETICS subroutine..

Table 3. New variables for Nitrogen Fixation

| Name | units | Description |
| --- | --- | --- |
| NFIX_OPTION_EXIST | --- | Logic switch indicating whether the N-fixation file “nfix_option_input.csv” exists, or not. |
| CRIT_TIN(1 ... NAL) | mg/L | User-input algal group-specific critical TIN concentration, below which N-fixation is triggered. |

The code changes for the alternative N-fixation capability do not impact the W2 model input control file (i.e., w2_con.npt or w2_con.csv). A separate comma-separated value input file (“nfix_option_input.csv”) controlling this logic is read by the model if the file exists in the same folder as the W2 model input control file. The inputs allow the user to specify values for CRIT_TIN corresponding to each algal group state variable being simulated by the model.

The content of the three-line input file (including header lines that can be used to describe the input variables and/or other user-specified information) is described as follows:

Alternate N-fixation option inputs for 1 to NAL algal groups (header)

CRIT_TIN(1), CRIT_TIN(2), ... CRIT_TIN(NAL)

2.0, 1.0,

New code. The code changes implemented for the alternative algal N-fixation option are described below. Code line numbers for each of the revised code files are included in the changes listed below

Declare new variables in code file w2modules.f90 –

Module KINETIC:

(314)	REAL,    ALLOCATABLE, DIMENSION(:)    :: CRIT_TIN

Module MAIN:

(613)	LOGICAL       :: NFIX_OPTION_EXIST

Allocate and apply new variables in code file input.f90 –

Allocate memory for arrays

(328)	ALLOCATE (CRIT_TIN(NAL)

Determine and/or read new variables in the “nfix_option_input.csv” input file

(2319)	!> sch 25Jan2025. Start reading N-fixation option inputs.

(2320)	  NFIX_OPTION_EXIST = .FALSE.

(2321)	  CRIT_TIN = 0.0

(2322)	  INQUIRE(FILE='nfix_option_input.csv',EXIST=NFIX_OPTION_EXIST)

(2323)	  IF(NFIX_OPTION_EXIST) THEN

(2324)	    OPEN(NUNIT,file='nfix_option_input.csv',status='old')

(2325)	    READ (NUNIT, *)

(2326)	    READ (NUNIT, *)

(2327)	    READ (NUNIT, *) (CRIT_TIN(JA), JA=1,NAL)

(2328)	    CLOSE(NUNIT)

(2329)	  ENDIF

(2330)	!> sch 25Jan2025. End reading N-fixation option inputs.

Implement alternate in “water-quality.f90” subroutine KINETICS –

When triggered, apply alternate N-fixation option so that there is no nitrogen limitation on growth for the corresponding algal group state variable. (Note: an equivalent code option would be to use ANLIM(K,I,JA) = 1.0 for line 279).

(278)	IF ((NH4(K,I)+NO3(K,I)) < CRIT_TIN(JA)) THEN

(279)	  ANLIM(K,I,JA) = (NH4(K,I)+NO3(K,I))/(NH4(K,I)+NO3(K,I)+0.0+NONZERO)

(280)	  ENDIF

Deallocate memory associated with new variables in code file endsimulation.f90 –

(305)	DEALLOCATE (CRIT_TIN)

### Testing

Demonstration of expected change in result from enabling feature.

### Additional Notes

- Continuous base N-fixation is an active research area. More complex options for handling base N-fixation may be scientifically supported in the future. Potential options include:

  - Monod: If TIN < Ccrit (two options)

    - modify half saturation from user entered value to zero proportionally (accommodates non-linear Monod)

    - Use Monod directly (modify existing substrate limitation Monod approach in W2 (e.g., N, P, Si, light limitation)

  - Base level and N-fixation (combination of above two approaches)

- The following files (no new code files are needed) would be edited and/or created to incorporate this capability:

  - input.f90 – reads run control input files “w2_ con.csv” or “w2_con.npt” and where the input variables (assuming a kinetics-modification approach is most appropriate in this case) needed to control this capability are read.

  - w2modules.f90 – non-local variable definitions and memory allocation

  - water-quality.f90 –  add code for base N-fixation calculations.

  - wqconstituents.f90 – add code to set a minimum algae standing crop (see low DO-high mortality option).

  - endsimulation.f90 – end of simulation memory deallocation.

  - Various time variable and snapshot option output code files (output.f90, outputa2wtools.f90 and outputinitw2tools.f90)  – may need to be modified for tracking when, where, duration and N mass additions from N-fixation (for each specific algae group to which it may be applied).

## References