# DRAFT REPORT: CE-QUAL-W2 Harmful Algal Bloom Prediction Tool Research

Prepared for: Dr. Jodi Ryder, USACE-ERDC-EL-WQB

Prepared by: LimnoTech

Contents

Overview	1

Background	2

Process	3

Literature Review	3

Internal Discussion	3

Interviews and Online Meetings	3

Outcomes	4

Next Steps	6

Appendix A: Participants	7

Appendix B: Online Meeting Notes	8

## Overview

Task 1 (“CE-QUAL-W2 Harmful Algal Bloom Prediction Tool Research”) of Delivery Order W912HZ23F0288 (“Water Quality Tools for Enhancing Natural and Agricultural Areas”) under USACE-ERDC Contract No. W912HZ-20-D-0004 called for LimnoTech to research existing Harmful Algal Bloom (HAB) simulation algorithms, considering algal species, vertical migration, and the factors that cause HABs. A recent literature review performed by ERDC-EL staff was found to be comprehensive in its scope, so research efforts were focused instead on elicitation of stakeholder views on HABs-related capabilities and needs with respect to the CE-QUAL-W2 (W2) two-dimensional hydrodynamic and water quality model. Elicitation was performed through interviews, a two-part online workshop, and extensive discussions between the LimnoTech-led contractor team and ERDC staff.

This note documents the elicitation process and outcomes and constitutes the Task deliverable. Potentially significant needs identified in the process were for:

- Code enhancements related to buoyancy, nitrogen cycling, and hypoxic conditions.

- Documentation of best practices for segmentation, etc.

- Identification, collection, and documentation of existing useful tools.

- Development of an effective and engaged developer and/or user community.

- Linkage to high-resolution datasets or models to superimpose heterogeneity on W2’s laterally-averaged results.

Useful suggestions outside of the direct scope of this effort included:

- Remote measurement of ambient toxin concentrations.

- Improvements to ERDC’s HAB Explorer web tool.

- Development of an integrated W2 GUI.

## Background

The U.S. Army Engineer Research and Development Center (ERDC), Environmental Laboratory (EL), Water Quality and Contaminant Modeling Branch (EPW) require tasks and deliverables for the development and application of predictive mathematical models to solve complex environmental challenges through multi-disciplinary integration of science and engineering. Outcomes of this research are to elucidate the ecological process issues that reduce the contamination of waterways and groundwater to protect the environment, fish, and wildlife.

One such challenge is the increasing prevalence of harmful algal blooms. Changes in land use and climate have contributed to an increased prevalence of elevated nutrient levels and temperatures in surface waters that are conducive to the development of algal blooms. These blooms can range from unsightly and malodorous to deadly to ecosystems and pets. These blooms can impede mechanical or chemical processes for drinking water treatment, hinder recreational activity, and can even bring disease or death to humans and animals.

The CE-QUAL-W2 (W2) model was developed by the Corps to improve simulate hydrodynamics and water quality in reservoirs, rivers, estuaries, lakes and river basin systems. It is capable of modeling eutrophication processes, including the interactions between temperature, sediment, nutrients, organic matter, algae, and dissolved oxygen, and has been applied to thousands of water bodies in the United States and around the world. Despite this broad foundation of model applications, W2’s utility for simulating HABs is not yet proven. This Task is intended to develop a better understanding of W2’s limitations and existing capabilities for modeling HABs; this will serve as a basis for identification, prioritization, and implementation of new W2 capabilities.

## Process

### Literature Review

The original scope of work for this task called for a literature review. ERDC staff noted that a literature review by ERDC tentatively titled “Current Capabilities and Opportunities for Numerical Water Quality Modeling of Harmful Algae Blooms (HABs)” was already in preparation; Contractor and ERDC review and discussion of the draft review document concluded that the ERDC document was comprehensive and current, and that Task efforts could therefore be focused on internal discussions and elicitation of stakeholder needs and perspectives.

### Internal Discussion

The contractor team, joined by ERDC staff as available, met multiple times to

- Review team understanding of W2 and its applicability to HABs;

- Form hypotheses about potential enhancements to W2 to improve its applicability;

- Identify likely stakeholders with useful perspectives; and

- Plan and coordinate the interviews and online workshop used for elicitation.

Three types of enhancements were provisionally identified during these meetings:

- Code modifications to add or improve W2 functionality with respect to key HABs processes;

- Documentation of best practices identified in real-world HABs applications of W2, such as fine vertical segmentation to capture diurnal movement.

- Linkage to higher-resolution datasets (e.g. remote sensing of algal mass distribution within a segment) or models (e.g. 3-D hydrodynamic circulation).

### Interviews and Online Meetings

Nearly 40 potential participants were identified, of whom 23 were able to contribute (Appendix A). Organizations represented include the US Army Corps of Engineers (USACE), Environmental Protection Agency (EPA), National Oceanic and Atmospheric Administration (NOAA), US Geological Survey (USGS), Bureau of Reclamation (USBR), and several universities.

An online workshop was developed to gather insights and identify opportunities for enhancing the W2 model to better address HABs in reservoirs. The workshop was divided into two 90-minute sessions: the first to identify existing tools and needs, and the second to prioritize new capabilities for W2.

- Session 1 Agenda

  - Welcome to webinar

  - Introduction

  - Comments on W2 capabilities

  - Facilitated discussion of needs and existing capabilities

  - Recap and next steps

- Session 2 Agenda

  - Welcome to webinar

  - Recap of Meeting 2

  - Discussion and Priortization

  - Next steps

Several mechanisms were used to facilitate participant engagement and contributions, including:

- Early publication of agenda and supporting materials

- Encouragement to use (where permitted) shared meeting documents (Appendix B)

- Rapid development and dissemination of meeting minutes

- Outreach to participants who were not able to attend the online meetings, and inclusion of their perspectives in supporting materials.

A team meeting was held after the first online session to confirm consensus on what was heard and to identify themes for the second session, and a final team meeting was held after the second online meeting to debrief on key received messages.

## Outcomes

Participants expressed a need for improvements in remote measurement of toxin concentrations, user-friendly web tools, and validation data on vertical stratification and mixing. High-frequency data, algae speciation information, and nutrient loading were highlighted as critical data needs. Specifically, there was a call for operational models that are easy to run, capable of short-term forecasting, and sensitive to the impacts of different water withdrawal regimes on HABs.

They also emphasized the necessity for W2 to be more user-friendly and efficient in calibration. Enhancements in model resolution and probabilistic outputs were suggested to improve decision-making. There was also interest in incorporating remote sensing data into W2 to better predict conditions at depth.

Clarity was sought on the real-world utility of W2 processes, the development of quick tools for immediate use, and how W2 can inform decision-making and action triggers. They also explored how W2 could work with remote sensing limitations and spatial data.

Additional detail was provided on several points:

- Data and Model Needs: The integration of multi-/hyper-spectral remote sensing data was discussed, emphasizing its value in providing spatial and temporal resolution. Ground-truthing was identified as necessary for reliable model-data comparisons.

- Ecosystem Impact Assessments: Participants debated the temporal limitations of W2 in assessing ecosystem impacts and considered the dual model approach used in the Central Valley.

- Model Dimensions: The feasibility of a 3D version of W2 was discussed but deemed impractical due to computational demands and data limitations. Instead, a sub-grid model or post-processing to extract lateral information from a 2D model was suggested.

- Wind Data: The challenge of obtaining spatially-resolute wind data for driving the model was highlighted, with suggestions to use remote sensing data as a surrogate.

- Buoyancy and Mixing: The team agreed to delve deeper into the representation of buoyancy and wind-driven mixing in the model.

- Model Utility: There was a consensus on the need to document the conditions under which W2 is useful and to ensure adequate data to constrain uncertainties.

- Predictive Capabilities: The role of W2 in predicting acute concentrations and its utility in alerting potentially critical conditions was discussed.

- Model Application and Calibration: The usefulness of W2 in organizing information and aiding decision-making was affirmed. The need for effective calibration tools, such as PEST and Ostrich, was emphasized.

- Integration with Remote Sensing: The potential for using satellite imagery to start and calibrate models was explored. Training data for identifying action triggers through machine learning was also considered.

- User Interface and Community Building: Suggestions included developing a true GUI for W2, improving visualization of time series data, and fostering a community for sharing insights and advancements.

The contractor team and ERDC staff concluded that the most important needs identified in the workshop were:

- Code enhancements related to buoyancy, nitrogen cycling, and hypoxic conditions - current model code will be reviewed and performance assessed to identify opportunities for improvement/enhancement.

- Documentation of best practices for the user community based on W2 expert applications – topics may include handling of segmentation, light extinction, algal communities, data collection, model suitability, and characterization and communication of uncertainty and risks.

- Identification, collection, and documentation of tools – an assortment of tools useful in applying W2 already exist and can be collected to the benefit of W2 users.

- Development of a W2 community - an effective and engaged developer and/or user community will turbocharge sharing of code enhancements, best practices, tools, and more.

- Linkage to high-resolution datasets or models – W2’s laterally-averaged model outputs may be sharpened through superposition of heterogeneity based on (e.g) hyperspectral imagery, 3-D hydrodynamics, or detailed wind fields.

The workshop concluded with added direction for enhancing the W2 model to better serve the needs of HABs management in reservoirs, leveraging both traditional modeling techniques and modern data integration methods.

## Next Steps

The outcomes of this task will be fed forward to Task 1 (“Design of Harmful Algal Bloom algorithms for CE-QUAL-W2.” of Delivery Order W912HZ24F0178 (2ERDC25) (“Water Quality Model Development to Manage Harmful Algal Blooms”).

Appendix A: Participants

| Organizers |  |  | Participants |
| --- | --- | --- | --- |
| Jodi Ryder - ERDC |  |  | Norm Buccola - USACE/Detroit Lake |
| Todd Steissberg - ERDC |  |  | Carl Nim - USACE/Allegheny |
| Lauren Melendez - ERDC |  |  | Marvin Boyer - USACE/Milford |
| Angela Poovey - ERDC |  |  | Zac Wolf - USACE/Harsha |
| Tad Slawecki - LimnoTech |  |  | Tony Clyde - USACE |
| Mike Deas - Watercourse |  |  | Isaac Mudge - USACE |
| Scott Hinz - LimnoTech |  |  | Mandy Micahlsen - USACE/ERDC |
| Billy Johnson - LimnoTech |  |  | Alyssa Eck - USACE |
| Todd Redder - LimnoTech |  |  | Richard Johansen - USACE/remote sensing |
| Dave Dilks - LimnoTech |  |  | Mick Porter - USACE |
|  |  |  | Brenda Rashleigh - EPA |
|  |  |  | Ben Cope - EPA |
|  |  |  | Chris Knightes - EPA |
|  |  |  | Stewart Rounds - USGS |
|  |  |  | Erik Smith - USGS |
|  |  |  | Natalie Hall - USGS |
|  |  |  | Drew Loney - USBR |
|  |  |  | Lindsay Bearup - USBR |
|  |  |  | Carolyn Owens - NOAA |
|  |  |  | Kunal Rathore - Oregon State (Machine Learning) |
|  |  |  | Scott Wells - Portland State |
|  |  |  | Zhonglong Zhang - Portland State/LimnoTech |
|  |  |  | Emily Summers - TAMU |

## Appendix B: Online Meeting Notes

# Online Workshop: HABs & W2

Online Workshop: HABs & W2

Meeting Link

Purpose

Participants

● Planning Group

● Invited Participants

Pre-Workshop Insights

Needs - from Registrants and Interviews

Other

Data

W2-specific(?)

Additional Questions - from Planning Group

Session 1

Agenda

Notes

[Dilks]

[Hinz]

[Natalie Hall]

5/24 Internal Debrief

Session 2

Agenda

Outcomes

## Meeting Link

https://limno.zoom.us/j/85043337070?pwd=esAuVnWbBF8yK6XdfY1F9LVSsp2pvQ.1

PASSCODE: 714559

## Purpose

ERDC is working with LimnoTech (contractor) to collect information about HABs modeling tools and needs for reservoirs in order to identify opportunities to extend CE-QUAL-W2’s capabilities  to meet those needs. One activity is a two-part online workshop, where the first session will focus on identifying existing tools and needs, and the second on identifying and prioritizing desirable new capabilities for CE-QUAL-W2. The workshop outcomes will inform future ERDC conceptualization and implementation activities.

This shared Google Doc is provided so that participants can (if access is available) review supporting materials and contribute additional material directly. Snapshots of the document will be sent to all participants for the convenience of those who do not have access to the online document.

## Participants

Because of time constraints, there will not be a slot for introductions in either session. This list of participants is included both for your perusal and to give you the opportunity to provide any relevant additional information you’d like others to know.

### Planning Group

  - Jodi Ryder - ERDC

  - Todd Steissberg - ERDC ® 
  Research Environmental Engineer, formerly a classical violinist, love to play ukulele, guitar, and other instruments

  - Lauren Melendez - ERDC

  - Angela Poovey - ERDC ®

  - Tad Slawecki - LimnoTech	
  Senior Engineer/Computer Manager

  - Mike Deas - Watercourse Engineering

  - Scott Hinz - LimnoTech ®
  green

  - Dave Dilks - LimnoTech

  - Billy Johnson - LimnoTech

  - Todd Redder - LimnoTech

### Invited Participants

  - Norm Buccola - USACE/Detroit Lake

  - Carl Nim - USACE/Allegheny

  - Marvin Boyer - USACE/Milford

  - Zac Wolf - USACE/Harsha

  - Tony Clyde - USACE

  - Isaac Mudge - USACE ®
  Water Resources Engineer, USACE-MVN

  - Mandy Micahlsen - USACE/ERDC

  - Alyssa Eck - USACE

  - Richard Johansen - USACE/ERDC ®
  Researcher, monitoring, remote sensing

  - John Kucharski - USACE/ERDC

  - Mick Porter - USACE

  - Brenda Rashleigh - EPA

  - Ben Cope - EPA

  - Chris Knightes - EPA ®

  - Caroline Owens - NOAA ®
  Quantitative Ecologist, prefer the color of no bloom

  - Stewart Rounds - USGS ®
  Hydrologist and Scientist Emeritus

  - Erik Smith - USGS ®
  Hydrologist, CE-QUAL-W2 modeler, dealing w/ HABs on mostly reservoirs

  - Natalie Hall - USGS

  - Jenny Olszewski - USGS

  - Drew Loney - USBR ®
  Wisconsin cheese is the best.

  - Lindsay Bearup - USBR ®
  Civil Engineer - Hydrologic; love to paddle board with no algal blooms of any color

  - Kunal Rathore - Oregon State (Machine Learning)

  - Scott Wells - Portland State ®

  - Zhonglong Zhang - Portland State/LimnoTech

  - Emily Summers - TAMU ®
  Just defended her PhD this month!

## Pre-Workshop Insights

This section will include any interesting insights gleaned from pre-workshop interviews and volunteered information from webinar registration entries.

### Needs - from Registrants and Interviews

#### Other

- Remote measurement of toxin concentrations would be nice. (x2)

- Make Molly’s web tool easier to use and accessible to the public … a dashboard with the most current map for each lake of interest rather than having to repeat a long sequence over and over.

#### Data

- Validation data on vertical stratification and mixing

- Higher-frequency data

- Algae speciation information, knowledge of factors contributing to HABs

- Nutrient loading

- Take advantage of hyperspectral RS.

#### W2-specific(?)

- Need an operational model that is ready and easy to run.

- Focus has been on toxins and public/human health -> Trophic cascades/ecosystem impacts on everything else

- Forecast short-term (14-day) algal conditions in the vicinity of discharge structures under different discharge scenarios to help with help with implementation of fish passage/temperature driven operations; also swimming safety on the upcoming weekend

- How might HABs change depending on withdrawal regime - mix of surface and deep water -> possible transport of cyanobacteria/nutrients/sediments downstream.(2x)

  - How to operate dam to best meet multiple objectives

  - W2 could be a useful tool if it doesn't take up so much time to calibrate and updated

- Make sure importance of wind pushing HABs towards discharge structures is captured

- Improved resolution - finer than Ohio EPA's toolset.

- Probabilistic/risk output similar to ORSANCO empirical model would be useful for communicating (tool opportunity?)

### Additional Questions - from Planning Group

- What model processes and model application workflows are useful in the real world?

- What can be done with "vanilla" W2? Are there some useful quick tools we can develop for use as soon as this summer?

- What can a W2 application give the manager to inform decision-making?

- How can W2 help identify action triggers?

- How useful is development of an action trigger based on a limited scenario - perhaps with a single dominant species and limited observations - to drive decisions?

- How can we work with remote sensing limitations - invisible tributaries, coarse resolution for most reservoirs except at infrequent occasions) - can W2 be used to interpolate/downscale?

- How can W2 best take advantage of spatial data, perhaps for drying-out cells like Shasta Marina?

## Session 1

Session 1 is focused on learning about HABs-related needs and tools to lay a foundation for identifying possible enhancements to CE-QUAL-W2 algorithms and application.

### Agenda

- Welcome to webinar (Tad Slawecki)

- Introduction(Todd Steissberg)

- Comments on W2 capabilities (Mike Deas)

- Facilitated discussion of needs and existing capabilities (All)

- Recap and next steps (Tad Slawecki and Mike Deas)

### Notes

#### [Dilks]

- Data needs

  - Is mutli-/hyper-spectral remote sensing data of value to W2?

    - Yes. Especially with spatial and temporal resolution. Need tools to convert remote sensing  data in a form that can be used by the model.

    - Some need for ground-truthing of remote sensing  data but model-data comparisons to date have been promising.

  - Would be a useful intersection to use remote sensing  data for the surface with W2 capabilities to predict at depth

- Model needs

  - Can W2 assess ecosystem impacts?

    - To some extent, but not necessarily at the temporal scale needed,

    - ResSim can do this to some extent, but is new and only 1-dimensional

    - Dual model approach being taken in Central Valley, W2 + 1-D ResSim

  - Need to assess localized impacts (e.g. boat ramp) at a spatial scale finer than can be directly addressed by W2.

  - Are there any thoughts of going to 3-D version of W2?

    - Ph.D. student at PSU is working on W3. 2-D model has outperformed 3-D version in all applications to date.Adding an extra dimension does not mean better predictive capacity.

    - Perhaps a sub-grid model or post-processing to tease lateral information from a 2D model.

    - Don’t have spatially-resolute wind data to drive “algae piling up on shore” problem with 3D model.

    - Can multi-/hyper-spectral remote sensing data be used as a surrogate for detailed wind data?

      - Might be feasible on large water bodies but not necessarily smaller reservoirs. Upper Klamath suggested as a test bed, but noted that it may be too wide for the assumption of laterally-averaged conditions to be valid..

    - PSU investigating using a modeled wind field as a forcing function to model

- Attendee by attendee comments

  - Can we think about different wind drivers depending on size of the lake

  - Consideration of phosphorus added via fire retardants applied for wildfires.

  - Would like to see videos of case study applications

  - Let’s address low hanging fruit first (e.g. optional buoyancy module); recognize that one size doesn’t fit all (e.g. W2 not great on wide systems)

  - Tulsa added a Habitat Suitability Index to W2, This provides utility that could be built upon to identify triggers.

#### [Hinz]

Getting more information from Scott Wells about how they are actually trying to implement 3D grids in W2 would be worthwhile (and also how he approaches setting up model-data comparisons w.r.t. 3D always looking worse). Stated mainly because there is no detail on website about it (just a reference to it being in development). If Mike is going to be talking with him, getting more detailed information would be nice.

In general, it seems not worth pursuing a 3D W2 in this case, at least in the short term, anyway. A 3D W2 would not be conducive as an operational management model (in most cases) due to the computational requirements (2D seems a bit iffy for that now in many cases). The EFDC model application on Lake Tenkiller was somewhat poor (as Scott Wells noted), partly due to the limited number of vertical layers and partly due to the overall grid coarseness. However, it was also partly just because of the type of vertical grid system employed at the time (the modelers may have actually later modified their EFDC code to allow for a different vertical grid system that they claim worked much better ... but 10 layers is still not enough and Scott Well's W2 model of the reservoir may have been more like 50 layers ... pretty much the standard minimum number that gets used ... and it really was much better than the EFDC model).

A few other things not specific to HABs per se or W2 (just eutrophication modeling in general):

- Nutrient loading was a data bullet, but not really discussed. note use of data-based (empirical) vs. watershed model-based loads. This is almost always a sticky point somewhere down the road regarding water quality calibration. Should everyone be on the same page about it?

- Algal speciation ... the model is always modeling assemblages that someone decides how to group. Assuming that is done well, it still seems clear that you'd need more than a single group of cyanobacteria (toxin producers vs non-toxin producers, other factors) and also enough other groups to be able to represent how a system might respond over time to changes in loads and other external forcing conditions. Modeling just 3 algal groups with respect to the level of confidence for forecasting might not be enough. It would be a modeler and management issue (and perhaps a data issue), not a model capability issue. Also, are there other algal processes that should be considered for possible incorporation as options, based on other advanced eutrophication models?

With regard to variable buoyancy, Mike and Stewart may have been talking about incorporating something a bit different than what W2 can now do.

Specific bullets that got a lot of discussion:

- remote sensing (with and without ground-truthing) useful for model-data comparisons ... to at least evaluate trends

- wind fields (data limitations, atmospheric forcing models [research stage], alternatively perhaps combine with W2 at relevant local spatial scales to inform decision making)

W2 can already output certain types of management-related information for dissolved oxygen, so similar things could likely be implemented for other model state variables and/or derived state variables without it being a huge effort.

Regarding the W2 capabilities that could perhaps be used as template for HABs ... it's what the user manual (Part 3) describes under "Fish Habitat Volumes". Something similar could be done, but using different types of "triggers" than temperature and DO (e.g., HABs toxins, Chl-a [group specific] and maybe still DO triggers) to generate similar types of outputs that might be useful for management purposes and/or used in conjunction with other factors for decision making. Noted because there are already some capabilities that someone might be able to build upon w.r.t. HABs.

#### [Natalie Hall]

nhall@usgs.gov]

Here are a couple of links re our HABs hyperspectral work (note that the data release includes both spectral identifications as well water quality measurements):

- Carl J. Legleiter, Tyler V. King, Kurt D. Carpenter, Natalie C. Hall, Adam C. Mumford, Terry Slonecker, Jennifer L. Graham, Victoria G. Stengel, Nancy Simon, Barry H. Rosen, Spectral mixture analysis for surveillance of harmful algal blooms (SMASH): A field-, laboratory-, and satellite-based approach to identifying cyanobacteria genera from remotely sensed data, Remote Sensing of Environment, Volume 279, 2022

https://www.sciencedirect.com/science/article/pii/S0034425722002036

|  | Spectral mixture analysis for surveillance of harmful algal blooms (SMASH): A field-, laboratory-, and satellite-based approach to identifying cyanobacteria genera from remotely sensed data Algal blooms around the world are increasing in frequency and severity, often with the possibility of adverse effects on human and ecosystem health. T… www.sciencedirect.com |
| --- | --- |

- Hall, N.C., Kishimoto, J.J., Shtabnoy, A., Legleiter, C.J., King, T.V., Mumford, A.C., Spaulding, S.A., Carpenter, K.D., and Slonecker, T., 2024, Hyperspectral profiles of Harmful Algal Blooms (HABs) and other algae, 2022: U.S. Geological Survey data release, https://doi.org/10.5066/P9AOGQBT

I would be happy to talk further about a possible collaboration. Current and ongoing efforts include:

- I am actively seeking samples for the hyperspectral microscopy work

- we are developing chlorophyll and phycocyanin machine learning models

- we are also developing calibration/validation protocols for field spectrometers and field data collections of HABs and other algae

[From chat]

[Stewart Rounds] - Upper Klamath Lake is a great system for looking at remotely sensed data, but the circulation is such that it is not a good candidate for CE-QUAL-W2.

[Caroline Owens] - [...] I'm not the most familiar with CE-QUAL-W2 but maybe a set of criteria (length/width ratio?) to quickly check whether it is appropriate would be useful if that doesn't already exist.

[Stewart Rounds] - a width/depth ratio may be a place to start.  Not sure.  But UKL has an interesting circulation pattern that is somewhat clockwise and specific.  That would be the biggest problem in using a 2D (longitudinal, vertical) model because the 3D movement is so important.

### 5/24 Internal Debrief

Billy, Jodi, Scott H, Tad, Todd S

- Look into buoyancy more deeply - unidirectional, circadian rhythms versus steady state

- Nitrogen/non-nitrogen fixers, how well are diatoms represented in W2

- 3D version of W2 quashed (for now) (computational needs, grid resolution and type, data needs, …)

- Characterizing near-shore/shallow water mixing <-> light limitations -> warmer surface with more nutrients, …

- Wind-driven mixing - complex-enough problem to put off for now, look for simpler site-specific expressions.

- Documentation from this effort should describe what the resultant product is good for, when is it useful.

- What question is being answered? Is there adequate data to constrain uncertainty (modeling first principles)?

- Can we predict acute concentrations? Risk of false positives;

  - can we downscale from W2 averaged segment to a “more critical” location, perhaps based on wind field or hyperspectral imagery?

- Role of model may be to alert of potentially critical conditions so that watchfulness (monitoring) is elevated.

- Screening

  - Should W2 be used at all?

  - If a W2 model exists (temperature? eutrophication?), is it going to be useful?

- Interfaces for high-resolution hyperspectral or wind field inputs - abstraction/upscaling?

- HABs volumes as analogy to Fish Habitat Volumes

- Communication between stakeholders and developers

- Versioning and anti-fragmentation

- Do Stewart’s visualization tools have anything useful for HABs? (Philadelphia District examples from Zhong)

## Session 2

Session 2 is intended to focus on confirmation and prioritization of possible enhancements to CE-QUAL-W2 algorithms and application.

### Agenda

- Welcome to webinar (Tad Slawecki)

- Recap

- Discussion

- Next steps

### Outcomes

How do people want to use W2? What is the end goal? Drew: performance for calibration - PEST; Scott Wells - Not necessarily effective; Drew: https://usbr.github.io/ostrich/index.html

Scott Wells - Modeling is useful for organizing information;

Erik Smith - calibrate W2 to Sentinel2 Chlorophyll-a; PEST is tricky

ChatGPT + PEST?

Pre- and post-processing tools - Stewart’s tool for limno+W2 W2-anim in github (perl+tcl/tk) + W2 post-processor (tecplot); Erik R calibration tools (Klamath) - Norm - https://github.com/nbuccola/w2r water balance calibration - CLEARINGHOUSE

Organize information -> give insights 

Brenda: Speak to toxins - can’t predict toxins

Norm - use satellite imagery to start/calibrate model for 1-2 week outlet -> phycocyanin

W2 as a part of the tool. Imagery -> algal density and light extinction inputs

Training data -> identifying action triggers through a neural network. May or may not be independent of W2.

Look at multiple sources - Landsat, Pace, Sentinel [CYAN], NASA-SBG, Planet;

Imagery filmstrip - check with MTRI? Google Earth Engine

When not to use W2

CYAN - 

Depth as a trigger

[Caroline] Central clearinghouse, extensions … where can we use W2

[Billy] - true GUI

[Drew] - Decouple engine from interface; calibration tools

[Erik] - Nitrogen fixation, W2 is easier to calibrate; Pull in and weight multiple weather stations

[Lauren] -  visualization of time series data

[Lindsay] - community

[Natalie] - HABs community change <- taxonomic data

[Norm] - Better UI - new GUI with Stewart NN

[Scott H] - need good information on species [Mike]; incorporating RS; Quality of nutrient loading forcing (modeled versus empirical)

[Stewart] - RS -> calibration; workshop on case studies with parameter sets.

[Scott W] - Interpolated/synthesized/downscaled weather is not necessarily good.

[Isaac] - Satellite data -